EXPOSURE TRIANGLE INTERACTIVE

Max Dupain Exhibition — State Library of NSW

Claude Code Build Brief — Phase 1: Browser Prototype

# 1. Context

A single-plinth interactive installation in a small projection room (~5m wide × 5–7m deep) at the State Library of NSW. One end wall carries a full-width short-throw projection. Visitors use three physical rotary encoders on a plinth to explore the photographic exposure triangle — manipulating a projected Dupain photograph in real time to discover the settings Dupain actually used.

The experience is game-like: values start randomised, visitor must 'find' the correct exposure. Feedback is visual and progressive.

# 2. Hardware

- 3× continuous-rotation rotary encoders (with detents) wired to Arduino

- 4–6× illuminated momentary buttons for image selection

- Arduino outputs clean serial over USB

- Zotac Mini PC, Nvidia 4070, Windows 11

- Full-wall projection, 1080p minimum

Serial Format

A:127,S:64,I:200,BTN:2\n

BTN field is only sent on press events — not continuously. BTN:0 is not transmitted.

Encoder Behaviour

Clamped accumulator — no wrapping. Clockwise increases value toward 255 (hard stop), counter-clockwise decreases toward 0 (hard stop). Physical knob continues to rotate freely at either extreme. Recommend increment of 3–5 per detent so full range traverses in ~1–2 rotations.

# 3. Image Assets

Per image, two files at matching resolution (1920×1080px):

- image_A.jpg — original photograph

- image_A_depth.jpg — greyscale mask: dark = near/sharp, white = far/blurred

Use placeholder grey images for prototype. Final Dupain assets substituted later.

Depth map convention: dark values are close to camera (stay sharp at wide aperture), white values are distant (receive blur). Shader uses (1.0 – depthValue) as blur weight.

# 4. Effect Chains

## 4.1 Aperture  (f/1.4 → f/16, encoder range 0–255)

- Primary: depth-masked Gaussian blur on image

- Wide open (0) = blurred background, sharp foreground — blur weight = (1.0 - depthValue) × maxBlur

- Stopped down (255) = sharp throughout — blur weight approaches 0

- Secondary: subtle exposure shift — wide open slightly brighter, stopped down slightly darker (~20–30% brightness range)

## 4.2 Shutter Speed  (1/15s → 1/500s, encoder range 0–255)

- Primary: radial blur from image centre

- Slow (0) = heavy radial blur (camera shake / handheld instability read)

- Fast (255) = clean, no blur

- Secondary: exposure shift — slow = overexposed/bright, fast = underexposed/dark (~moderate range, between ISO and aperture in intensity)

## 4.3 ISO  (64 → 12800, encoder range 0–255)

- Primary: film grain overlay — grain intensity and apparent grain size both scale with value

- Secondary: dramatic exposure shift — low ISO (0) = underexposed/crushed blacks, high ISO (255) = overexposed/blown toward white

- This is the most dramatic exposure effect of the three — nearly full brightness range

| Control | Primary Effect | Exposure Shift | Intensity |
| --- | --- | --- | --- |
| Aperture | Depth-masked blur | Subtle | ~20–30% range |
| Shutter Speed | Radial blur (centre) | Moderate | ~50% range |
| ISO | Film grain | Dramatic | ~full range |

# 5. Triangle HUD

## 5.1 Layout

An equilateral triangle drawn in thin lines (~1px). One vertex per parameter: Aperture, Shutter Speed, ISO. Each edge has a single dot that travels between the two endpoint vertices as the encoder value changes. Small numeric readout beside each dot showing current value in photographic units (f/2.8, 1/60s, ISO 400 etc.).

## 5.2 Behaviour

- Idle (no interaction for 60s): HUD invisible

- First encoder touch: HUD fades in smoothly

- Active: dots move along edges in real time

- Approaching correct value per axis: dot begins to glow amber

- All three correct: triangle pulses/completes (brief bright flash or fill), shutter click fires, text overlay appears

- After trigger: HUD fades back to dim ready state

## 5.3 Visual Style

- Thin lines, low opacity baseline (suggest ~25–35% opacity at rest)

- Cool grey/white colour at rest

- Amber glow on dots as they approach target

- Sparse — no fills, no backgrounds, small type only

- Position: suggest lower-centre of frame, clear of main subject

# 6. Correct Exposure Logic

## 6.1 Data Structure

Each image has a defined target window for all three parameters:

{

id: 'dupain_01',

image: 'image_01.jpg',

depth: 'image_01_depth.jpg',

label: 'f/11 · 1/125s · ISO 100 — Dupain\'s choice for hard midday light',

targets: {

aperture:  { value: 200, tolerance: 20 },   // encoder units

shutter:   { value: 180, tolerance: 20 },

iso:       { value: 60,  tolerance: 20 }

}

}

## 6.2 Proximity Score

Each axis produces a normalised score 0–1 where 1 = within tolerance window. Combined score = average of all three. Threshold bands drive HUD feedback states:

- Score < 0.5: no feedback, image looks wrong

- Score 0.5–0.8: HUD dots begin warming to amber

- Score > 0.8: amber glow intensifies

- All three within tolerance simultaneously: trigger correct-exposure event

## 6.3 Trigger Logic

Include hysteresis — once triggered, requires all values to move outside tolerance before re-triggering. Prevents rapid-firing if a value hovers on the boundary.

## 6.4 Correct Exposure Event Sequence

- Shutter click audio sample plays

- Triangle HUD pulses / completes

- Text overlay fades in: image label string (e.g. 'f/11 · 1/125s · ISO 100 — Dupain\'s choice for hard midday light')

- Hold 4 seconds, fade out

- HUD returns to dim ready state

# 7. Idle / Attract State

If no encoder input for 60 seconds:

- HUD fades out

- Image values slowly drift away from correct exposure in a looping animation — not random, but a gentle predictable oscillation so it reads as intentional

- On next encoder touch: drift stops, current encoder position takes over, HUD fades in

# 8. Image Selection

- 4–6 illuminated momentary buttons on plinth

- Each button maps to one Dupain image

- On press: image crossfades to new selection, values randomise to new start position within 'plausibly wrong but not absurd' ranges

- Randomise within mid-range bands — avoid starting at extremes (e.g. not ISO max + aperture wide open simultaneously)

# 9. Fallback UI (No Hardware)

Toggle with keypress (suggest 'H' for hardware/software toggle):

- Three on-screen sliders replace encoder input

- Buttons replaced by clickable image thumbnails or number keys 1–4

- Sliders labelled with photographic units, not raw 0–255 values

- Visual indicator showing current mode (Hardware / Simulator)

# 10. Deliverables

- Single HTML file — runs in Chrome, fullscreen, WebGL shaders, Web Serial API

- Arduino sketch — clamped encoder accumulator, button-on-press serial output, correct format

- PORTING_NOTES.md — documents exact maths, data structures, trigger logic, idle timing, serial format for TouchDesigner port

# 11. Code Style

- Single HTML file — keep everything together

- Clean, well-commented code — this is a prototype, not production

- WebGL shader code must be readable and annotated inline

- Avoid over-engineering

- WebGL is required — no Canvas2D (must run at full projection resolution without frame drops)

- Placeholder assets: solid grey images acceptable for initial build

# 12. PORTING_NOTES.md Requirements

The markdown file must document the following clearly for TouchDesigner implementation:

- Exact mathematical relationship: encoder value (0–255) → each effect parameter

- Correct exposure window data structure per image

- Trigger logic including hysteresis

- Idle state timing logic

- Serial message format from Arduino

- Depth map convention (dark = near, white = far, shader inversion)

- HUD triangle geometry and dot position calculation